Politecnico di Milano
Dipartimento di Scienze e Tecnologie Aerospaziali
Prova finale: Introduzione all’Analisi di Missioni Spaziali
Docente: Massari Mauro

Elaborato n. C13

Autori:

| 10694997 | Pala Silvia |
| --- | --- |
| 10711624 | Turcu Alex Cristian |
| 10730510 | Vanelli Paolo |

Anno Accademico 2022-2023

Data di consegna: 00/00/00

# Table of contents

Table of contents	2

1.	Introduction	3

2.	Initial orbit characterization	3

2.1.	Initial orbital parameters	3

2.2.	Data interpretation	3

2.3.	Graphical representation	4

3.	Final orbit characterization	5

3.1.	Final position and velocity	5

3.2.	Data interpretation	5

3.3.	Graphical representation	6

4.	Transfer trajectory definition and analysis	6

4.1.	Standard maneuver	6

4.2.	Alternative maneuvers	6

4.3.	Graphical representation	6

5.	Conclusions	6

6.	Appendix	7

# Introduction

The present project aim is to study, optimize and choose various orbital transfer strategies, having as initial data a point on the initial orbit, which position and velocity vectors are given, and a point on the final orbit, which is defined by its orbital parameters.

First it will be analysed a strategy based on a set of standard maneuvers.

Several other alternative strategies have been examined to try to optimize the two most significant parameters in their distinction: the maneuvering cost (the total speed gap required to complete all the orbital changes) and the operating time (from the start point to the final point).

# Initial orbit characterization

## Initial orbital parameters

The assigned starting position and velocity vectors are the following:

ri=-1169.7791-8344.5289977.8062 km      vi= 4.2770-1.9310-4.9330 kms

Using MATLAB software, it is possible to calculate the orbital parameters assigned to this specific couple of vectors:

| ai [km] | ei | ii [deg] | ωi [deg] | Ωi [deg] | θi [deg] |
| --- | --- | --- | --- | --- | --- |
| 83679 | 0.1097 | 48.6244 | 67.8907 | 87.8870 | 103.2729 |

## Data interpretation

The starting geocentric orbit is elliptical, with an eccentricity value between 0 and 1 and a specific energy of:

Ei= -μ2ai= -23.8119 km2s2

It belongs to Medium Earth Orbit (MEO) category, as its apogee and its perigee are inside the range of 8000 – 42000 km:

rai= 9288 km

rpi=7452 km

According to the given value, it is nor a polar nor a geo-synchronous orbit and has a period of:

Ti=7620 s=2 h, 7 m, 0 s

## Graphical representation

A graphical representation of the orbit is possible with the aid of MATLAB software:

# Final orbit characterization

## Final position and velocity

The goal orbit, that is geocentric just like the starting one, is defined by its orbital parameters:

| af [km] | ef | if [deg] | ωf [deg] | Ωf [deg] | θf [deg] |
| --- | --- | --- | --- | --- | --- |
| 10860 | 0.2332 | 30.2751 | 24.6315 | 173.2051 | 18.9993 |

The final position and velocity are calculated from these parameters using again the MATLAB software:

rf=-6640.6-4258.22927.0 km      vf= 4.2742-5.57982.9393 kms

## Data interpretation

The final geocentric orbit is elliptical, with an eccentricity value between 0 and 1 and a specific energy of:

Ef= -μ2af= -18.3517 km2s2

It belongs to Medium Earth Orbit (MEO) category, as its apogee and its perigee are inside the range of 8000 – 42000 km:

raf=13393 km

rpf=8327 km

According to the given value, it is nor a polar nor a geo-synchronous orbit and has a period of:

Tf=13423 s=3 h, 43 m, 43 s

## Graphical representation

# Transfer trajectory definition and analysis

## Standard maneuver

## Alternative maneuvers

## Graphical representation

# Conclusions

# Appendix